Flight Control System:

Overview:

The purpose of the flight control system is to provide reliable indoor flight for a quad-rotor vehicle when a characterized/reliable position estimate is available, and when a set of desired locations/poses are commanded.

This system will accept a desired position in map space, given a North-West-Up coordinate system (used as per autopilot specifications), as well as a yaw command. These commands will be sent from the planning subsystem.  Additionally, the SLAM algorithm described in the perception subsystem will provide position and orientation estimates to be used in the estimators used for control.

These commands and estimates are sent to an onboard Pixhawk autopilot. The autopilot uses these estimates and commands to run it’s internal control algorithms, which interface directly with the motors and ESCs. The Pixhawk uses a successive loop closure approach with position control as the outer most loop and the inner loops consisting of velocity, attitude, and attitude rate controllers.

Modeling:

Summary of Work

A simulation was developed to model the behavior of a quadcopter given certain physical properties and position commands. The simulation consists of a position controller, an attitude controller, and quadcopter dynamic model. The FlyNet simulation was built on the framework developed by D. Hartman, et al. (2014) and modified to fit the needs of this particular project and quadcopter. An overview of the model is shown in Figure 1.

Figure 1. FlyNet quadcopter Simulink model

The simulation requires three sets of inputs:

- Initial conditions for all of the states (P, Q, R, ϕ, θ, ψ, U, V, W, X, Y, Z) and motor angluar rates, ωmotor

- Physical parameters of quadcopter (for a full list see Table 2)

- Path commands (X, Y, Z, and ψ)

These inputs are used by the model to simulate the quadcopter dynamics given a list of waypoints.

Position Controller

The position controller uses PD to control the desired velocity in the body horizontal plane (Xb and Yb). The desired velocity is derived from the error between desired and current inertial positions. Figure 2 shows Simulink diagram of the position controller.

Figure 2. FlyNet quadcopter simulation position controller

The position controller outputs a signal bus containing roll and pitch attitude commands in addition to the altitude and yaw commands provided by the path command. This signal bus is then passed to the attitude controller.

Attitude Controller

The attitude controller applies PID control to the roll, pitch, and yaw in addition to the body Z coordinate in order to maintain the attitude required to execute the path command provided to the model. The attitude controller Simulink model is shown in Figure 3.

Figure 3. FlyNet quadcopter simulation attitude controller

The attitude controller uses body angular positions and rates (P, Q, R) in addition to the roll, pitch, and yaw commands provided by the position controller to compute the attitude corrections necessary to maintain a given path. The attitude corrections are sent to the motor mixer in order to translate them in to motor RPM commands. The motor mixer is shown in Figure 4.

Figure 4. FlyNet quadcopter simulation motor mixer

The variable ‘mc#’ refers to the individual motor RPM commands that are required to achieve the attitude corrections. At this time, motor dynamics are considered and modeled in this simulation according to the transfer function shown in Equation (1):

|  | ΩV=ktτms+1 | (1) |
| --- | --- | --- |

Here, Ω is the motor angular velocity, V is the PWM signal provided to the motor, kt is the motor torque constant, and τm is the motor time constant. Once the motor commands are known, the dynamic model can simulate the response of the quadcopter to external force and moments.

Quadcopter Dynamics

Quadcopter dynamics are well understood and documented in various papers. Detailed information on the physics behind the quadcopter dynamic model used in this simulation can be found in the work of D. Hartman, et al. (2014). It is important, however, to note that the current dynamics model ignores blade flapping and frame aerodynamic drag. Because the quadcopter velocities are assumed to be small, these effects are negligible.

Simulation Results

The simulation was first tested using step input commands for X, Y, and Z to simulate the behavior of quadcopter in response to 1 meter commands in each direction. Figure 5 shows the response of the quadcopter in the X and Y directions (the test yielded the same result for both directions).

Figure 5. X and Y response of model to step command

In the X-Y direction, the quadcopter shows a stable response with an overshoot of 14% with a 5% settling time of 3.7 seconds. The altitude (Z direction) step response exhibits slower performance but is still stable. Figure 6 shows the model response to an altitude step command.

Figure 6. Z response of model to step command

In the Z direction, the quadcopter shows a stable response with an overshoot of 50% with a 5% settling time of 22.4 seconds. The altitude control is less critical than the horizontal control for the purposes of the FlyNet mission because minimal altitude control is expected. Horizontal control, however, is pivotal to maneuvering obstacle-filled spaces.

Yaw angle is another command that was tested because it will be necessary to perform yaw maneuvers in order to scan the search area during the mission. Figure 7 shows the model response to a yaw step command.

Figure 7. Yaw response of model to step command

The yaw angle control is stable, showing an overshoot of 25% with a 5% settling time of 6.6 seconds. These results show the controller is stable and performs well in horizontal motion but should be improved in the other direction. Improvement can be achieved by further tuning PID gains or adding feedforward control.

In order to assess the quadcopter model’s ability to follow a path, a series of position commands were provided to the model. The path shown in Table 1 was provided to the model.

Table 1. Simulink model commanded path

|  | 0 min | 6 min | 7 min | 7.5 min | 8 min | 8.5 min | 9 min | 9.5 min | 10 min |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X [in] | 126 | 165 | 84 | 50 | 46 | 48 | 130 | 280 | 270 |
| Y [in] | 30 | 179 | 329 | 362 | 497 | 584 | 548 | 467 | 480 |
| Z [in] | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 |

The XYZ commands were given at the time intervals shown. The first command was given 6 minutes to execute as it was the initial change in altitude from ground level to 36 inches (3 feet). After the first command, constant altitude was maintained. Figure 8 and Figure 9 shows the simulated results of the quadcopter path following.

Figure 8. Top-down view of simulated quadcopter path-following

Figure 9. Isometric view of simulated quadcopter path-following

The results show that after an initial overshoot in altitude, the vehicle tends to adhere to the commanded path. There are slight deviations when heading is changed but that is expected based on the simulated controller step responses. The simulation shows a Root-Mean-Square (RMS) error of 8.50 and the requirement is RMS must be less than 20 for path following.

Table 2. Quadcopter simulation physical parameters.

| Variable | Parameter | Units |
| --- | --- | --- |
| g | Acceleration due to gravity | ms2 |
| d | Distance between motor and the axis of rotation | m |
| mass | Total mass | kg |
| ct | Lumped parameter thrust coefficient | Ns2rad2 |
| cq | Lumped parameter torque coefficient | Nms2rad2 |
| Jx | Moment of inertia about X | kg∙m2 |
| Jy | Moment of inertia about Y | kg∙m2 |
| Jz | Moment of inertia about Z | kg∙m2 |
| Jm | Moment of inertia of motors | kg∙m2 |
| Jb | Inertia tensor | kg∙m2 |
| Jbinv | Inverted inertia tensor | kg∙m2 |
| dctcq | Matrix of ct and cq for moment calculation | Ns2rad2, Nms2rad2 |
| motor_m | Mass of one motor | kg |
| motor_dm | Distance from motor to quadcopter center of mass | m |
| motor_h | Height of motor | m |
| motor_r | Radius of motor | m |
| ESC_m | Mass of one ESC | kg |
| ESC_a | Width of ESC | m |
| ESC_b | Length of ESC | m |
| ESC_ds | Distance from ESC to center of mass | m |
| HUB_m | Mass of hub | kg |
| HUB_r | Radius of hub | m |
| HUB_H | Height of hub | m |
| arms_m | Mass of one arm | kg |
| arms_r | Radius of arm | m |
| arms_L | Length of arm | m |
| arms_da | Distance from center of mass to arm | m |
| minThr | Minimum throttle | % |
| T | Time constant | s |
| cr | Throttle % to RPM conversion coefficient | RPM%Throttle |
| b | Y-intercept for linear regression model of cr | RPM |

Hardware:

A 3DR Pixhawk autopilot is being used as the low-level flight controller and state estimator.

The Pixhawk internally uses accelerometers built into the unit to estimate its orientation. It also has a built in barometer for altitude state estimation; however, this is not being used since the barometer estimates are unreliable even over a short time frame. The magnetometer onboard the Pixhawk can also provide yaw estimates.

Figure 10: Pixhawk Autopilot

The PX4 firmware is being used in our system. This firmware is convenient for this application because it has built in support for external pose estimates. This allows the use of VICON for pose estimates during the controller performance evaluations, initial flight-testing for waypoint following, and for testing of the planning subsystem. Additionally, this topic will be used in the final flight configuration, in which a SLAM solution provides the pose estimates.

INSERT FIRMWARE VERSION HERE

Interfaces:

The communication between the planning subsystem, perception subsystem, and the flight control subsystem is handled using the Robot Operating System (ROS). The MAVROS library allows direct communication with the PX4 firmware running on the Pixhawk. This allows the team to set parameters on board the Pixhawk easily, and even during flight which is extremely convenient for tuning controller and estimator gains and other flight parameters.

Additionally, the MAVROS interface is used to publish pose estimates and pose commands to the internal Pixhawk topics. The following topics are used in MAVROS:

INSERT LIST OF TOPICS HERE

INSERT MAVROS VERSION HERE

Coordinate Systems:

The following list of coordinate systems is relevant to the control system:

- VICON: (???)

- Pixhawk: (North-East-Down) the coordinate system used by the Pixhawk internally (???)

- MAVROS: (North-West-Up) the coordinate system that is expected when passing position commands and estimates to MAVROS topics.

- SLAM: (???)

Results:

Figure 10: X Position Waypoint Tracking (Pose Estimate: Blue, Setpoint: Green)

After the coordinate system transformations were discovered and implemented in ROS nodes used to convert between coordinate systems, a successful flight test was performed that tested the waypoint tracking capabilities of the system.

Figure 11: Z Position Waypoint Tracking (Pose Estimate: Blue, Setpoint: Green)

Figure 11: Y Position Waypoint Tracking (Pose Estimate: Blue, Setpoint: Green)

These results demonstrate the successful development of a test bed for future work on the integration of planning and perception algorithms.

Future Work:

The future work for this subsystem includes the further tuning of control gains, and the integration of the subsystem with perception algorithm pose estimates. Once this system is integrated with the SLAM system, further tuning of control gains will be needed in order to compensate for the SLAM position estimate performance, which at this point is being characterized.